## Step 1 – Prepare a Farm Map and Calculate Available Land for Spreading

Prepare a farm map by identifying areas where manure and slurry must not be spread and areas where spreading poses a risk of pollution and calculate the hectares in these areas. The map used may be an IACS Map, Google Map or Ordinance Survey Map. Blue – Draw all water sources, courses, ponds, lochs, marsh or bog areas on the farm map.

| Color-Coded Risk Level | Details | Total Area (ha) |
| --- | --- | --- |
| Red – No Spread Zone | - Within 50m of a spring, well or borehole. - 10m of any ditch, burn, river, loch or wetland. - Slopes in excess of a gradient of 11° (1 in 5). - Shallow soils with average soil depth of less than 40cm and overlies gravel or fissured rock. - Areas under separate management agreements such as SSSI and agri-environment schemes. - Unimproved land that has not previously received slurry or manure. - Wooded land, steading, roads and yards. |  |
| Orange – High Risk Area | - Slopes with a gradient of between 8 and 11° (approx. 1 in 7 to 1 in 5). - Areas at risk of flooding more often than one in five years. - Fields with drainage installed in the previous 12 months. - Poorly drained, waterlogged or severely compacted land. |  |
| Yellow – Moderate Risk Area | - Slopes with a gradient of between 4 and 7° (approx. 1 in 14 to 1 in 8). - Land sloping towards watercourses or water supplies. - Imperfectly drained or saturated land. |  |
| Green – Low Risk Area | - Slopes with a gradient up to 3° (approx. 1 in 19). - Land with no artificial drainage. |  |
|  | Total Available Land Ha (Orange + Yellow + Green) (A) = |  |

## Step 2 – Area Required Calculation

Calculate the area required to spread the manure produced by calculating the minimum area required:

| Stock Unit | No of Stock on Unit | Months Housed | Hectares needed By Stock Unit | Total Area Needed (Ha) |
| --- | --- | --- | --- | --- |
| Cow (650 kg) |  | x | X 0.039 | = |
| Cow (550 kg) |  | x | X 0.032 | = |
| Cow (450 kg) |  | x | X 0.025 | = |
| Heifer 2 years + (500 kg) |  | x | X 0.019 | = |
| Youngstock 1-2 years (400 kg) |  | x | X 0.016 | = |
| Youngstock 6-12 months |  | x | X 0.008 | = |
| Calf |  | x | X 0.005 | = |
| Bull |  | x | X 0.019 | = |
| Sheep |  | x | X 0.003 | = |
| Lamb (up to 6 months) |  | x | X 0.001 | = |
| Lamb (6-12 months) |  | x | X 0.002 | = |
|  |  |  | Total Area Required (B): |  |

## Step 3 – Mass Balance

| Total Available Land (A) | Total Area Required (B) | Balance (A - B) |
| --- | --- | --- |